The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

A high resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/ep17htda-2.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Die Attach Epoxy Featuring High Tg Meets NASA Low Outgassing Requirements

Master Bond EP17HTDA-2 is a one component epoxy that can be used for bonding, sealing and die attachment. It is not premixed and frozen, and cures with heat. This dimensionally stable system features excellent mechanical properties with both a high modulus and die shear strength. It transfers heat and resists thermo-mechanical stresses effectively.

EP17HTDA-2 has exceptional temperature resistance with a service temperature range of -80°F to +600°F [-62°C to +316°C] and a Tg of 185-190°C. This compound has a thermal conductivity of 9-10 BTU•in/ft2•hr•°F [1.30-1.44 W/(m·K)], a low coefficient of thermal expansion, and is also electrically insulative with a volume resistivity greater than 1015 ohm-cm. It withstands a variety of chemicals including acids, bases, salts, fuels, oils, water and many solvents.

As a one part system, EP17HTDA-2 does not require any mixing and is curable in the temperature range of 300-350°F for around 4-5 hours. To optimize properties, a post cure of 2-3 hours at 400°F is recommended. It bonds well to a wide variety of substrates, such as metals, ceramics, plastics and composites. Upon curing, it delivers a tensile modulus exceeding 1,100,000 psi and a die shear strength of 20-23 kg-f at room temperature (2 x 2 mm [80 x 80 mil]). It also has minimal shrinkage upon curing.

While EP17HTDA-2 is well suited for electronic and related applications, it can also be used in vacuum situations as it passes NASA low outgassing testing. It is a thixotropic paste, yet is available in 10 cc and 30 cc syringes for automated dispensing, and has a shelf life of 3-6 months when stored at 40-50°F.

Master Bond Die Attach Adhesives

Master Bond EP17HTDA-2 is a one part epoxy system for die attach, bonding and sealing applications that features high temperature resistance, thermal conductivity, electrical insulation and NASA low outgassing approval. Read more about Master Bond’s die attach adhesives at https://www.masterbond.com/industries/die-attach-epoxy-adhesives or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/ep17htda-2

Check out new videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #