The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/ep41s-5med .

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Non-Cytotoxic Epoxy Withstands Multiple Methods of Medical Sterilization

Master Bond EP41S-5Med is a two part epoxy system that meets the requirements of ISO 10993-5 for non-cytotoxicity and can be used for bonding, sealing, and coating in medical devices. It features high temperature resistance and good strength properties. As an electrically insulative material, it can be utilized in medical electronic applications.

This epoxy was tested against many sterilization methods, such as autoclaving, STERRAD®, Cidex® and Steris®, successfully resisting a hundred cycles. It is also capable of withstanding exposure to gamma radiation, ethylene oxide, as well as other aggressive sterilants and chemicals. EP41S-5Med provides a tensile strength of 10,000-12,000 psi and adheres well to a variety of similar and dissimilar substrates, including metals, plastics, rubbers, ceramics, and composites. This compound has a glass transition temperature of 140-145°C and is serviceable from -80°F to +450°F [-62°C to +232°C]. EP41S-5Med features reliable electrical insulation characteristics with a dielectric constant of 3.5 at 60 Hz and a volume resistivity greater than 1014 ohm-cm.

The system is black in color and contains no solvents or diluents. EP41S-5Med offers convenient handling with a 100 to 25 mix ratio by weight. The mixed epoxy has good flow properties with a moderate viscosity of 25,000-30,000 cps.  Upon cure, it offers low shrinkage and a hardness of 85-95 Shore D.  EP41S-5Med comes in standard sized units of ½ pint, pint, quart, and gallon kits. It is also compatible with automated gun dispensing systems.

Master Bond Biocompatible Adhesives

Master Bond EP41S-5Med is a high temperature and chemically resistant epoxy that meets ISO 10993-5 specifications. This two part, high tensile strength system can withstand exposure to repeated autoclaving and aggressive sterilants. Read about Master Bond medical grade adhesives at https://www.masterbond.com/properties/biocompatible-adhesives or contact technical support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/ep41s-5med

Check out new videos on our YouTube channel: https://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #