The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at http://www.masterbond.com/newsrelease/mastersil-912med.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

One Component, Neutral Curing Silicone Passes USP Class VI and ISO 10993-5 Tests

Specially formulated for medical device applications, Master Bond MasterSil 912Med is a room temperature vulcanizing silicone system for high performance bonding, sealing and coating. This system meets both USP biocompatibility and ISO cytotoxicity standards and resists a variety of sterilization techniques such as gamma radiation, EtO and some types of liquid sterilants.

This one part system has a paste viscosity and non-corrosive cure with a tack-free time of 15-30 minutes at room temperature. Similar to other single component silicones, MasterSil 912Med’s cures depended upon the humidity level. The higher the humidity, the faster the cure and thinner sections cure more quickly than thicker ones. It is translucent in color and is serviceable over the wide temperature range of -75°F to +400°F.

MasterSil 912Med has a tensile strength of 700-1,100 psi and offers superior adhesion to many materials, including metals, composites, ceramics, glass, rubbers, plastics and other silicone substrates. Featuring an elongation of 250-350%, this flexible system can withstand highly rigorous thermal cycling as well as thermal and mechanical shock. It is a reliable electrical insulator with a volume resistivity of >1015 ohm-cm.

MasterSil 912Med is available in 80 gram tubes and has a shelf life of 6 months.

Master Bond Biocompatible Adhesives

MasterSil 912Med is high temperature resistant, flexible one part silicone that complies with USP Class VI and ISO 10993-5 requirements. This sterilization resistant formulation also features impressive electrical properties. Read more about Master Bond’s medical grade adhesives at http://www.masterbond.com/properties/biocompatible-adhesives or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: http://www.masterbond.com/tds/mastersil-912med

Check out new videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #