The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

A high resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/supreme-112.

-----------------------------------------------------------------

FOR IMMEDIATE RELEASE

High Tg, Ultra Low Viscosity Epoxy meets NASA Low Outgassing Specifications

Master Bond Supreme 112 is a new two part epoxy for bonding, sealing, potting, and impregnation applications. It is a heat curable system with a very low viscosity of 50-200 cps and a long open time of 2-3 days at room temperature for a 100 gram batch. Supreme 112 features a high glass transition temperature of 190-195°C and is serviceable up to 288°C (550°F). This compound bonds well to a variety of substrates including metals, composites, glass and ceramics. As a toughened system it withstands thermal cycling and mechanical stresses.

“Supreme 112 can be applied successfully not only in bonding applications, but it can also be used in porosity sealing. Additionally, it offers a high tensile strength of 11,000-12,000 psi”, says Rohit Ramnath, Senior Product Engineer. “This non-solvent based compound also meets NASA low outgassing requirements.”

Supreme 112 retains its electrical insulation properties even at elevated temperatures. While this system has a relatively easy to use mix ratio, 100:70 by weight, it requires a three step oven curing process of 1-2 hours at 200°F, followed by 3-4 hours at 250°F and then 4-6 hours at 300°F. Despite the long cure schedule, Supreme 112 offers a unique combination of performance properties advantageous in many applications in aerospace, electronic and automotive industries.

Master Bond Electrically Insulative Epoxies

Master Bond Supreme 112 is a two component heat curing epoxy system with an ultra low viscosity and long working life. It offers high bond strength and reliable electrical insulation across a wide range of temperatures. Read more about Master Bond’s electrically insulative epoxy adhesives at https://www.masterbond.com/properties/electrically-insulative-adhesive-systems or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

About Master Bond

Since its founding in 1976, Master Bond has been committed to developing epoxies, silicones, light curing systems and other specialty adhesives that meet specific customer requirements. Master Bond manufactures over 3,000 grades of specially designed formulations that are widely used across a variety of industries including electronic, medical, aerospace and optical.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/supreme-112

Check out new videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #