-----------------------------------------------------------------

The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at http://www.masterbond.com/newsrelease/uv22dc80.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Nanosilica Filled, Dual Cure Adhesive Meets NASA Low Outgassing Specifications

Master Bond UV22DC80 is a one component, low viscosity, nanosilica filled epoxy based system for bonding, sealing and coating with the ability to cure under UV light and/or with heat. The UV portion of the cure can be achieved in seconds upon exposure to a UV light at 365 nm with 20-40 milliwatts/cm2 of energy. This type of formulation allows for polymerization in “shadowed out” areas with the addition of heat at 80°C for 40-60 minutes. This advantageous low temperature is useful when working with plastics or other heat sensitive substrates.

UV22DC80 is optically clear with a refractive index of 1.52 and passes NASA low outgassing testing per ASTM E595. It is a competent electrical insulator and features a volume resistivity exceeding 1014 ohm-cm. These properties make this compound well suited for high tech applications in aerospace, optical, optoelectronics and related industries.

As a nanosilica filled system, UV22DC80 delivers superior abrasion resistance, low shrinkage upon cure and dimensional stability. It bonds well to a wide variety of substrates, including metals, glass, ceramics and many plastics. UV22DC80 is a high strength product featuring a tensile strength of 6,000-8,000 psi. This system also resists chemicals such as acids, bases, fuels and solvents.

UV22DC80 has a service temperature range of -60°F to +350°F [-51°C to +177°C]. It can also withstand rigorous thermal cycling. This system has a glass transition temperature of 90°C, which can be increased to over 125°C if post cured at 125-150°C for 15-30 minutes. UV22DC80 is available for use in standard packaging in sizes ranging from ½ pints to 5 gallon containers and in convenient syringe dispensers.

Master Bond Dual Cure Adhesives

Master Bond UV22DC80 is an easy to handle, one part dual UV/heat cure system that meets NASA low outgassing specifications. It features excellent physical strength and temperature resistance properties. Read more about our dual cure adhesives at http://www.masterbond.com/products/dual-curing-adhesive-systems or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: http://www.masterbond.com/tds/uv22dc80

Check out more videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #