-----------------------------------------------------------------

The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at http://www.masterbond.com/newsrelease/uv25.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Dimensionally Stable UV Curable System with a High Glass Transition Temperature

Suitable for high production applications, Master Bond UV25 offers fast fixture times, rapid curing and high temperature resistance. This multifunctional one component UV curable system was developed for bonding, sealing, coating and encapsulation applications. It is widely used in aerospace, fiber-optic, optical, electronic and related industries.

Master Bond UV25’s most notable feature is its thermal stability, with a glass transition temperature of 186°C. It is serviceable over the broad range of -60°F to +500°F. Additionally, this material is optically clear with a refractive index of 1.55 at room temperature.

Upon exposure to a UV light source emitting at a wavelength of 320-365 nm with an energy output of 20-40 milliwatts per cm² UV25 cures in 20-30 seconds. The rate of cure also depends upon the compound’s distance from the light source, the thickness of the section and the intensity of the light source. It is not oxygen inhibited and therefore does not require any special treatment in that regard. The product does not contain any solvents or volatiles and features low shrinkage upon curing. It bonds well to glass, surface treated metals and plastics such as polycarbonates and acrylics, among others.

When being used in potting applications, this system can cure in section up to 0.100-0.120 inches. At room temperature, UV25 has a moderate viscosity of 7,000-11,000 cps. It is also a competent electrical insulator.

UV25 can be stored for up to 6 months in its original, unopened containers with no exposure to light. It is available in syringes, ½ pint, pint, quart, gallon and 5 gallon containers.

Master Bond High Glass Transition Temperature Adhesives

Master Bond UV25 delivers the convenience of a one part, no mix system and rapid cures upon exposure to UV light. It features high temperature resistance and optical clarity with excellent light transmission properties. Read more about Master Bond’s high glass transition temperature formulations at http://www.masterbond.com/properties/epoxy-adhesive-systems-high-glass-transition-temperature or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit:http://www.masterbond.com/newsrelease/uv25

Our video on UV25 is available here: http://www.masterbond.com/newsrelease/uv25

Check out more videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #